Names:

GIKUNDIRO Denyse           26RP00964

NZEYIMANA Nelly Stella   26RP00818

Program:

B-Tech in Electronics and Telecommunication Technology

RQF level: 8

Department:

Electrical and Electronics Engineering

Option/Trade: Electronics and Telecommunication Technology

Module code: ETTSE801

Module name: SMART EMBEDDED SYSTEM

Credits number: 15

Academic year: 2026/2027

Semester: I

DATE: 24/10/2026

Classroom Air Quality Monitoring & Machine Learning System

1. Executive Summary & Project Architecture

The Classroom Air Quality Monitoring & Machine Learning System is an end-to-end IoT platform engineered to evaluate, classify, and visualize indoor environmental health parameters in academic environments. Poor indoor air quality directly correlates with decreased cognitive focus, fatigue, and increased transmission of airborne pathogens. This system integrates edge computing, decision tree machine learning, real-time microcontroller telemetry, interactive web dashboards, and public GitHub repository tracking.

Hardware & Physical Sensing Layer

Temperature & Humidity:  DHT11 digital capacitive sensor tracking ambient classroom temperatures (0°C to 50°C ±2°C) and relative humidity levels (20% to 90% RH ±5%).

Gas & VOC Measurement:  MQ135 semiconductor gas sensor detecting airborne Volatile Organic Compounds (VOCs), Carbon Dioxide (CO2), ammonia, benzene, and smoke via analog voltage outputs digitized by a 10-bit ADC (0 to 1023 raw scale).

Edge Microcontroller:  Microcontroller processing unit (ESP8266 NodeMCU / ESP32) reading sensor voltages, performing edge inference, and transmitting serial stream logs at 115200 baud.

2. Exploratory Data Analysis & Statistical Profiling

The underlying machine learning dataset (air_quality_data.csv) comprises 1,500 continuous environmental telemetry records collected across varied operational classroom conditions. The target metric classifies indoor air quality into three distinct categories: Good, Moderate, and Poor.

| Feature | Unit | Min | Mean | Max | Std Dev | Data Range |
| --- | --- | --- | --- | --- | --- | --- |
| Temperature | °C | 20.50 | 29.33 | 37.50 | 4.82 | Ambient Range |
| Humidity | % | 33.80 | 66.66 | 99.00 | 18.45 | Relative Humidity |
| Gas (MQ135) | ADC (0-1023) | 45.00 | 506.45 | 1023.00 | 284.12 | Analog Gas Volts |

The dataset maintains perfect class balance across all three outcome states, containing exactly 500 samples each for 'Good', 'Moderate', and 'Poor' labels, eliminating class imbalance bias during algorithm training.

3. Machine Learning Model & Hyperparameter Specifications

The predictive pipeline is built in Python using Scikit-Learn (v1.8.0). Supervised multi-class classification is executed using a DecisionTreeClassifier. The fitted model parameters were inspected from air_quality_model_2.joblib:

Criterion:  Splitting criterion set to 'gini' impurity for recursive binary node splitting.

Max Depth:  Constrained to a depth of 4 levels to prevent overfitting and guarantee low latency during edge C++ inference.

Node Splitting Rules:  Minimum sample split requirement of 2; minimum leaf samples set to 1.

Model Persistence:  Serialized via joblib (NumpyArrayWrapper format with 64-bit float precision).

4. Microcontroller C++ Implementation & Header File

To achieve latency-free offline decision-making on low-power microcontrollers without cloud connectivity, the Scikit-Learn decision tree rules were transpiled directly into standard C++ syntax (arduino_tree.h). Below is the compiled conditional logic:

// Header File: arduino_tree.h // Transpiled Scikit-Learn Decision Tree Classifier for ESP8266/ESP32 Edge Inference const char* predictAirQuality(float gas, float temperature, float humidity) { if (gas <= 361.50) { if (temperature <= 29.95) { if (humidity <= 68.60) { return "Good"; } else { return "Moderate"; } } else { if (gas <= 250.00) { return "Good"; } else { return "Moderate"; } } } else { if (gas <= 652.50) { if (humidity <= 80.35) { return "Moderate"; } else { return "Poor"; } } else { if (humidity <= 62.30) { return "Moderate"; } else { return "Poor"; } } } }

Decision Threshold Analysis

Good Baseline Threshold:  Gas levels strictly ≤ 361.50 ADC units represent clean baseline conditions. If temperature is under 29.95°C and humidity under 68.60%, air quality is guaranteed 'Good'.

Moderate Threshold:  Gas readings between 361.50 and 652.50 ADC units denote elevated particulate/VOC presence, resulting in 'Moderate' labels unless high humidity (>80.35%) shifts the class to 'Poor'.

Poor Quality Trigger:  Gas levels exceeding 652.50 ADC units represent severe VOC concentrations, immediately triggering a 'Poor' quality status when humidity rises above 62.30%.

5. Firmware Execution & Serial Telemetry (air_quality_monitor.ino)

The Arduino firmware manages physical hardware synchronization, sensor signal processing, and diagnostic output streaming:

MQ135 Sensor Warmup:  Includes a non-blocking 120-second heater element stabilization delay for the MQ135 sensor upon system boot.

Noise Filtering:  Executes a 10-sample loop per measurement cycle to calculate moving-average ADC readings, filtering out transient electrical noise.

Serial Communication:  Configured for high-speed serial data streaming at 115200 baud to feed live telemetry to edge computing dashboards.

6. Streamlit Dashboard & Web Interface (dashboard.py)

The user-facing monitoring layer is powered by a high-performance Streamlit application integrated with Plotly graphic engines:

Multi-Source Data Ingestion:  Real-time mode ingestion supporting physical USB Serial streams from microcontrollers, interactive simulation mode, manual parameter sliders, and batch CSV file processing.

Visual Analytics Engine:  Interactive Plotly gauges for MQ135 ADC readings, dual-axis line charts for historical temperature and humidity trends, and multi-dimensional scatter plots mapping environmental features against predicted health tiers.

Automated Visual Alerts:  Real-time visual danger banners ('Poor Air Quality Detected: Initiate Classroom Ventilation / Open Windows') triggered when air quality degradation is predicted.

CSV Log Exporting:  Live session telemetry can be logged and exported into standardized CSV files directly from the browser context.

7. Web Application Deployment & GitHub Ecosystem

To support cloud access, remote monitoring, collaborative development, and reproducible research, the project software stack and associated machine learning repositories are hosted across public version control and cloud deployment platforms.

Live Streamlit Web Application

The interactive machine learning application and prediction portal is hosted live in the cloud, allowing remote users, school administrators, and researchers to interact with models without local Python installations:

Live Streamlit App URL:  Direct application URL: https://ctgugctdejdtnhl3jpy9p2.streamlit.app/

Cloud Architecture:  Runs on a cloud container engine with automatic redeployment upon repository commits, utilizing Streamlit Cloud hosting infrastructure.

GitHub Code Repository

The complete project source code, including firmware binaries, Jupyter notebooks, model pickle files, Streamlit dashboard scripts, and training datasets, is open-sourced and managed under Git version control:

GitHub Repository URL: https://github.com/gikundirodenise14-dotcom/air-quality-OG-/tree/main

Repository Features:  Includes full commit history, modular directory structure, dataset dependencies, requirements.txt, and C++ header integrations.